.get /users/bart/helios/letters/macros

.normal

.font 2

%cHelios Support for the Synergy Board

.sp 3

.font 54

%cBart Veer, 25.1.89

.np

.justifiedtext

.par

This document describes how to access the Synoptics Synergy board from

Helios. The document consists of three parts :the synserv.exe program;

the synlib.c interface library; and the low-level protocols used

between the library and the server.

.sp 2

.ti 0

The Synserv.exe Program

.par

The synserv.exe program is a special version of the Helios input/output

Server running on the host processor,

and takes the place of server.exe in the standard Helios system.

In addition to the usual facilities it provides a /synergy device,

which can be accessed by the interface library to interact with the

synergy board. The synergy board can only be accessed if there are

two lines in the server configuration file specifying the base address

of the synergy board and the file to be used when booting up the

board. Typical entries are:

.code

.sp

synergy_base = 0x2e0

synergy_file = synergy.cod

.sp

.justifiedtext

This specifies that the board is located at address 0x2e0 in the PC, the

default address, and that the file synergy.cod contains the code provided

by Synoptics which runs on the board.

.par

The synserv program does not boot up the board automatically, because

it may be desirable at times to reboot the transputer network without

rebooting the synergy board. However, the synergy board can be booted

via a call in the interface library, or by some other program run before

synserv.exe .

.sp 2

.ti 0

The Interface Library

.par

The synserv.exe program provides an additional Helios server called

/synergy, which may be accessed by programs running in the transputer

network. The protocols understood by this server are described below,

and hence the server may be accessed directly by application software,

but to facilitate programming an interface library is provided. This

library provides the same facilities as the standard synergy.cod program

provided by Synoptics, plus some extensions.

The library is written in C and it is assumed

that application software is also written in C. However, the library

may be called by any other language obeying the Helios calling

conventions, e.g. Fortran, and you are referred to the appropriate

manual for details of how to access C routines from other languages.

.par

The library routines provided are described below. All the routines

return 0 to indicate failure and non-zero to indicate success. The

library routines are defined in the header file syndef.h, which should

be #included in application sources. Note that most of the routines are the

same as those provided by Synoptics, and the reader is referred to the

Synoptics software manual for further details.

.sp 2

.table

.tableentry

int syn_Init(void)

.sp

This routine should be the first library routine called by the application

program. It initialises the library.

.tableentry

int syn_Tidy(void)

.sp

This routine should be called when the application finishes. It tidies

the interface library.

.tableentry

int syn_BootBoard(void)

.sp

This routine attempts to reboot the synergy board with the code

specified by the synergy_file option in the server configuration file.

.tableentry

int syn_Restart(void)

.sp

This routine attempts to execute the restart command on the

synergy board.

.tableentry

int syn_Vector(int x_off, int y_off, int width, int operand, int opcode)

.sp

This routine executes a vector operation. The operand should be

one of op_LowerByte, op_UpperByte, or op_EntireWord. The opcode should

be one of op_plus, op_minus, op_and, op_xor, op_times, op_divide,

op_put, op_get, op_swap, op_remap, or op_copy. The buffer should be

set up in advance using a syn_WriteBuffer() call.

.tableentry

int syn_Scalar(int x_off, int y_off, int width, int height, int operand,

int opcode, int value).

.sp

This routine executes a scalar operation on the window specified. The

operand should be one of op_LowerByte etc. as before, and the operation

should be one of op_plus etc.

.tableentry

int syn_Statistics(int x_off, int y_off, int width, int height,

int operand, int *buffer)

.sp

This routine executes a gather-statistics command on the synergy

board, reads the results, and stores these results in the buffer specified.

This buffer should be an array of at least 10 integers.

.tableentry

int syn_Histogram(int x_off, int y_off, int width, int height, int operand,

int *buffer)

.sp

This routine executes a histogram command on the synergy board and reads

the results into the buffer specified. This buffer should be an array of

at least 256 integers.

.tableentry

int syn_Convolution(int x_off, int y_off, int width, int height,

int patch_size, int type, int *buffer)

.sp

This routine performs a convolution. The patch-size should be 3, 5, 7, or

9, and the buffer should be an appropriately-sized array of integers

used for the convolution.

.tableentry

int syn_Zoom(int x_off1, int y_off1, int width1, int height1, int x_off2,

int y_off2, int width2, int height2_

.sp

This routine interpolates from one window to another window.

.tableentry

int syn_LoadInputLUT(int lut, int *buffer)

.sp

This routine loads the input lookup table. The lut should be in the

range 0-3, and the buffer should contain the 256 integers to be

used.

.tableentry

int syn_LoadOutputLUT(int luts, int *buffer)

.sp

This routine loads the output lookup table. If bit 3 of the luts argument

is set then the buffer should contain 256 integers to be loaded into

all three guns. If bit 3 is not set then bits 0, 1, and 2 determine

which one(s) of the red, green or blue guns will be loaded, and the buffer

should contain 256, 512 or 768 integers as appropriate.

.tableentry

int syn_LoadOverlayLUT(int *buffer)

.sp

This routine loads the overlay lookup table. The buffer should contain

9 integers.

.tableentry

int syn_RedefineCharacter(int ch, int *buffer)

.sp

This routine may be used to redefine a character. The ch argument specifies

the character to be redefined and should be in the range 0-255. The

buffer should contain 8 integers defining the new character.

.tableentry

int syn_String(int x_off, int y_off, int plane, char *text)

.sp

This routine draws some text at the location specified within the

corresponding plane in overlay memory. The text consists of a C string,

i.e. a number of bytes terminated by a single '\0' character.

.tableentry

int syn_CopyBlock(int x_off, int y_off, int plane, int width, int height,

int *buffer)

.sp

This routine performs the Synergy copyblock operation which I do not

understand.

.tableentry

int syn_Scroll(int x_off, int y_off, int plane, int width, int height,

int y_lines, int value)

.sp

This routine scrolls a window within overlay memory by y_lines, replacing

any cleared pixels with the value specified.

.tableentry

int syn_Wipe(int x_off, int y_off, int plane, int width, int height, int value)

.sp

This routine sets a window within overlay memory to the value specified.

.tableentry

int syn_Point(int x_off, int y_off, int plane, int value)

.sp

This routine sets a pixel in overlay memory to a particular value.

.tableentry

int syn_Line(int x_off, int y_off, int plane, int x1_off, int y1_off,

int value)

.sp

This routine draws a line within overlay memory between the coordinates

specified.

.tableentry

int syn_StartTVGrab(void)

.sp

This routine causes the framestore to start digitising the incoming

tv signal. The status of the grab may be inspected by a syn_ReadStatus()

call, and the grab should be terminated by a syn_StopTVGrab() call before

attempting to execute any other commands on the synergy board.

.tableentry

int syn_StopTVGrab(void)

.sp

This routine terminates a TV grab.

.tableentry

int syn_TVFilter(int x_off, int y_off, int width, int height,

int frame_delay, int A_coef, int B_coef)

.sp

This routine causes the framestore to start digitising the incoming

tv channel using the filter parameters specified. The status of the

grab may be inspected by a syn_ReadStatus() call, and the grab must be

terminated by a syn_FilterStop() call before attempting to execute any

other commands on the synergy board.

.tableentry

int syn_ReinitTV(void)

.tableentry

int syn_SetTVReplication(int display_base, int y_display)

.tableentry

int syn_SetSlowScanFilterCoef(int a_coef, int b_coef)

.tableentry

int syn_SetFilterMode(int movie, int sequence, int lut, int channel)

.tableentry

int syn_SetFilterType(int ignored_pixel, int ignored_line, int type,

int mode)

.tableentry

int syn_TVFormatSelect(int standard)

.sp

These routines control the various TV and filter parameters.

.tableentry

int syn_FilterStart(int x_off, int y_off, int width, int height, int type,

int host)

.sp

This routine is used to commence filtering the slow scan input. The status

of the filter may be inspected may the syn_ReadStatus() call, and the

filter must be terminated by a syn_FilterStop() call before attempting

to execute any other command on the synergy board.

.tableentry

int syn_FilterStop(void)

.sp

This routine is used to stop filtering the slow scan input.

.tableentry

int syn_CopyVideoMemory(int x_off, int y_off, int width, int height,

int x1_off, int y1_off, int src_type, int dest_type)

.sp

This routine is used to copy a block of video memory.

.tableentry

int syn_PhotoOutput(int x_off, int y_off, int width, int height, int host)

.sp

This routine is used to output a window to the slow-scan output port.

.tableentry

int syn_Execute(int code, ...)

.sp

This routine may be used to execute a command on the synergy board. The

command code should be one of com_Restart etc, as defined in the header

file syndef.h . This command code should be followed by any additional

arguments required. Note that if the command involves reading from or

writing to the buffer, this must be done explicitly using the

syn_ReadBuffer() and syn_WriteBuffer() calls.

.tableentry

int syn_ReadStatus(int amount, int *buffer)

.sp

This routine may be used to read the specified number of integers from

the synergy board's status buffer and store them in the buffer specified.

.tableentry

int syn_ReadBuffer(int amount, int *buffer)

.sp

This routine may be used to read the current values held in the synergy

board's video buffer. The specified number of integers are read and

stored in the buffer specified.

.tableentry

int syn_WriteBuffer(int amount, int *buffer)

.sp

This routine may be used to write some data to the synergy board's

video buffer. The specified number of integers are written.

.tableentry

int syn_WriteVideo(int first_line, int no_lines, int operand, char *data)

.sp

This routine can be used to transfer a large amount of data from transputer

memory to the synergy board's video memory. The first_line argument

specifies the offset of the first scan line, in the range 0-511. The

number of scan lines is specified by the second argument. The operand

argument should be one of op_LowerByte, op_UpperByte, or op_EntireWord.

The last argument specifies the address of the buffer in transputer memory.

If the operand is lower or upper byte the buffer should contain

(768 * no_lines) bytes, otherwise the buffer should contain

(768 * no_lines) short 16-bit integers.

.tableentry

int syn_ReadVideo(int first_line, int no_lines, int operand, char *data)

.sp

.endtable

This routine is the inverse of syn_WriteVideo(), and may be used

to transfer a large amount of data from synergy board video memory to

transputer memory.

.sp 2

.ti 0

The Library-Server Protocol

.sp

.par

The /synergy device is a standard Helios server obeying the General Server

Protocol. Hence an application opens a stream to the server, reads and

writes to the streams, seeks within the stream to move position, and

closes the stream when finished. The various different parts

of the synergy board's memory are mapped onto different positions within

the /synergy server, as follows:

.table

.tableentry

0x00000000 : the command buffer

.tableentry

0x10000000 : the status buffer

.tableentry

0x20000000 : the video buffer

.tableentry

0x30000000 : video memory treated as words

.tableentry

0x40000000 : video memory, top byte only

.tableentry

0x50000000 : video memory, bottom byte only.

.endtable

.sp

For example, consider the syn_LoadInputLUT() call in the interface library.

This routine calls syn_WriteBuffer() to transfer the lookup table to

the video buffer, and syn_WriteBuffer() performs a file seek to

offset 0x20000000 and then writes 256 words at that offset. The server

receives this seek request and hence knows that the next read or write

refers to the video buffer, and when it receives the write request it

transfers the data to the video buffer. The syn_LoadInputLUT() routine

then calls syn_Execute(), which seeks to offset 0x00000000 and then writes

data consisting of the command and its arguments. The synergy server knows

that a write to offset 0x00000000 is actually a request to execute a

command, so it checks that the synergy board is able to execute this

command i.e. that it is not busy scanning, interrupts the board to inform

it that it should execute the command, and then waits for the synergy

board to finish.

.par

Reading from or writing to video memory is similar. The interface library

seeks to the appropriate offset within the server. For example if the

application needs to write 20 lines

to video memory starting at scan line 10,

and needs to write to the top_byte only, then the library seeks to

offset (0x40000000 + (10 * 768)) and writes (20 & 768) bytes of data.

The server is responsible for transferring this data to the

appropriate bits of video memory.

.par

The server is intelligent enough to be able to distinguish between

ordinary commands and scanning commands. Hence it will not wait for

a scanning command to finish, and it will not allow the execution of

commands other than stop-scanning whilst a scan is in progress.

.sp 2

.ti 0

An example program

.par

The code below is a typical example program running on the transputer.

It restarts the board, assumed to be booted already, reads in all of

the picture held in the top byte of video memory, rotates it through

180 degrees within transputer memory, and writes it back to video memory.

.code

/*=========================================================================

===          Synergy Interface Test Program                             ===

===                                                                     ===

===     Copyright (C) 1989, Perihelion Software Ltd.                    ===

===                All Rights Reserved                                  ===

===                                                                     ===

===     Author : BLV, 13.1.89                                           ===

=========================================================================*/

#include <stdio.h>

#include <stdlib.h>

#include <helios.h>

#include "syndef.h"

#define eq ==

#define ne !=

#define failwith(a) { fprintf(stderr, a); return(1); }

int main(void)

{ BYTE *buffer;

int i, temp;

BYTE *start, *end;

if ((buffer = malloc(512 * 768)) eq (BYTE *) NULL)

failwith("Failed to allocate buffer.\n")

if (!syn_Init())

failwith("Failed to initialise interface library.\n")

if (!syn_Restart())

failwith("Failed to restart board.\n")

printf("Reading picture.\n");

if (!syn_ReadVideo(0, 512, op_UpperByte, buffer))

failwith("Failed to read video memory.\n")

printf("Picture read.\n");

start = buffer; end = (buffer + (768 * 512) - 1);

for (i = 0; i < (768 * 256); i++)

{ temp = *start; *start++ = *end; *end-- = temp; }

printf("Picture inverted.\n");

if (!syn_WriteVideo(0, 512, op_UpperByte, buffer))

failwith("Failed to write video memory.\n")

printf("Picture written.\n");

if (!syn_Tidy())

failwith("Failed to tidy interface library.\n")

free(buffer);

printf("Success.\n");

return(0);

}